4040 s2

Stereo

Integrated

Amplifier

SEPARATION: Disc input measured at 1 kHz -55dB

CD                                        -58dB

The deceptively simple exterior design of the 40 series conceals thoroughbred circuitry, which has been skilfully developed over the last few years by Creek engineers to give the highest possible performance for the least possible number of components. The purpose of this is to eliminate all unnecessary complication, giving a reproduction of sound with as little distortion of the signal as possible, or in other words, resulting in an output signal which is virtually identical to the original.

For those occasions when the original signal is imperfect, the 4040s2 is equipped with tone

The 4040s2 packs a punch. In its latest form it is capable of 40 W into 8 Ohm one channel driven, and will hold up well into more difficult loads. The build quality of the Series 2 is the highest yet. Low leakage electrolytics, polystyrene, and metallized stacked film capacitors are used in its signal paths. 20 Amp power transistors are clamped to a large finned heatsink that protrudes from the back panel, giving better dissipation for the increased power of the amp. The output circuitry is equipped with full over-voltage and current protection circuitry, making it virtually immune from outside abuse.

OUTPUT IMPEDANCE	0.15fl

TONE CONTROLS: Range at 20Hz, 20kHz ± 10dB

INPUTS	Sensitivity	Loading

Disc (MM)	2mV	47KI2	60pf

Tuner	250mV	35KO	25pf

OUTPUT

CD	500mV	22Kfl	25pf

from tape	10Vmax	1KS2

DISC EQ. ERROR	20Hz-20kHz ±C.5dB